COMAS ELECTRONICS

Thivon 458 - Ilion 13121

Athens, Greece

tel. +30 210 5769113, mob. +30 6977327021, fax: +30 210 4633823

email: support@comaselectronics.com

Concerning the AIS Recording in the BlueOcean SVDR

The BlueOcean SVDR can record target data from an on board AIS.

The recording of the AIS data is according IEC61996-2 4.6.8. All target data are recorded in their original form in order to be able to test the fidelity of the recording: the comparison of the original and recorded data is straightforward and the surveyor can easily determine that the recording is error-free, in accordance to IEC61996-2 4.4.3, Recording Integrity, point c (bit error rate).

The player equipment is available to investigation authorities according to IEC61996-2 5.5.3 d. Note that the player equipment is not recommended to be freely available for security reasons.

It is possible to have the AIS data in human readable form using a program by our company (available upon request). The decoding of the AIS data is according to IEC61993.

The use of the program is simple:

-	Select Mode = AIS, other setting as below.

-	Select the files to convert. Use the button at the right side of the Input File text box to find the file. AIS files have names similar to N2009011308591142.txt. “N” denotes AIS file and the rest is the date and time of file creation.

-	Select an output file name. We recommend to use a similar name, for example N2009011308591142a.txt, where a simple a is added to the filename.

-	Press “convert”.

The output file will contain human readable text, similar to this example:

13/01/2009 08:59:13.85  *  AIS               *  AIVDM,237429200 under way         ROT=731,38 SOG=     0  23o34' 0'' E   37o57' 0'' N  COG=  335 Heading=N.A.

13/01/2009 08:59:13.90  *  AIS               *  AIVDM,372201000 Aground           ROT=731,38 SOG=    10   N.A.           N.A.         COG=  360 Heading=N.A.

13/01/2009 08:59:13.92  *  AIS               *  AIVDM,352286000 under way         ROT=731,38 SOG=    10   N.A.           N.A.         COG=  360 Heading=N.A.

13/01/2009 08:59:13.95  *  AIS               *  AIVDM,237023700                   ROT=731,38 SOG=     1  23o37' 0'' E   37o56' 0'' N  COG=   71 Heading=N.A.

13/01/2009 08:59:13.98  *  AIS               *  AIVDM,319650000 under way         ROT=731,38 SOG=     0  23o40' 0'' E   37o56' 0'' N  COG=   30 Heading=N.A.

13/01/2009 08:59:14.29  *  AIS               *  AIVDM,

13/01/2009 08:59:14.70  *  AIS               *  AIVDM,237007000 under way         ROT=731,38 SOG=     0  23o37' 0'' E   37o56' 0'' N  COG=    0 Heading=N.A.

13/01/2009 08:59:14.73  *  AIS               *  AIVDM,305014000 under way         ROT=2880,0 SOG=     0  23o35' 0'' E   37o57' 0'' N  COG=    0 Heading=347o

13/01/2009 08:59:15.07  *  AIS               *  AIVDM,237098000                   ROT=731,38 SOG=     0  23o32' 0'' E   38o 1' 0'' N  COG=  212 Heading=N.A.

13/01/2009 08:59:15.20  *  AIS               *  AIVDM,239642000 restricted        ROT=731,38 SOG=     0  23o29' 0'' E   38o 1' 0'' N  COG=   64 Heading=N.A.

13/01/2009 08:59:15.32  *  AIS               *  AIVDM,237556100 under way         ROT=731,38 SOG=     0  23o31' 0'' E   38o 2' 0'' N  COG=   71 Heading=N.A.

13/01/2009 08:59:15.67  *  AIS               *  AIVDM,237023700                   ROT=731,38 SOG=     1  23o37' 0'' E   37o56' 0'' N  COG=   71 Heading=N.A.

13/01/2009 08:59:15.95  *  AIS               *  AIVDM,372201000 Aground           ROT=731,38 SOG=    10   N.A.           N.A.         COG=  360 Heading=N.A.